scheduling

RESETTING THE SIMULATION STATE

reset

-----

The 'reset' command will reset the simulation state to its initial

conditions and invoke each enabled element with the 'RESET' action in

order for them to place themselves back to the initial condiction state.

reset_elements path

-------------------

Using the 'reset' command gives no control over the

order in which the elements are 'RESET'. This is generally not a problem

if the initial state of one element does not depend on the initial state

of another element.

If there is such a dependency then the order in which elements are reset

is important and the more specific reset commands must be used.

NOTE: the simulator does not check for such dependencies. It is up to the

user to determine whether reset order is important. Individual objects

should document the conditions under which these dependencies arise.

For instance,

The object which computes the Nernst equilibrium potential requires

ionic concentrations. The initial potential therefore relies on the initial

ionic concentrations. Thus the elements which compute the ionic

concentrations must be reset before the elements which compute the

concentration-dependent equilibrium potentials.