_#P_#a_#c_#k_#e_#t _#D_#r_#i_#v_#e_#r _#P_#r_#o_#t_#o_#c_#o_#l

G. L. Chesson

Bell Laboratories

_#A_#b_#s_#t_#r_#a_#c_#t

These notes describe the packet driver link proto-

col that was supplied with the Seventh Edition of UNIX|#-

and is used by the UUCP program.

_#G_#e_#n_#e_#r_#a_#l

Information flow between a  pair  of  machines  may  be

regulated  by  first  representing  the  data  as  sequence-

numbered _#p_#a_#c_#k_#e_#t_#s of data and then  establishing  conventions

that  govern the use of sequence numbers.  The _#P_#K, or _#p_#a_#c_#k_#e_#t

_#d_#r_#i_#v_#e_#r, protocol is a particular instance of  this  type  of

flow-control  discipline.   The  technique  depends  on  the

notion of a transmission _#w_#i_#n_#d_#o_#w to determine upper and lower

bounds  for  valid  sequence  numbers.   The  transmitter is

allowed to retransmit packets having sequence numbers within

the  window  until  the receiver indicates that packets have

been correctly received.  Positive acknowledgement from  the

receiver  moves  the  window; negative acknowledgement or no

acknowledgement causes retransmission.   The  receiver  must

ignore  duplicate  transmission,  detect  the various errors

that may occur, and inform the transmitter when packets  are

correctly or incorrectly received.

The following paragraphs describe the  packet  formats,

message exchanges, and framing used by the protocol as coded

in the UUCP  program  and  the  UNIX  kernel.   Although  no

attempt will be made here to present internal details of the

algorithms that were used, the checksum routine is  supplied

for the benefit of other implementors.

_#P_#a_#c_#k_#e_#t _#F_#o_#r_#m_#a_#t_#s

The protocol is defined in terms of  message  transmis-

sions  of  8-bit  bytes.   Each message includes one _#c_#o_#n_#t_#r_#o_#l

byte plus a _#d_#a_#t_#a _#s_#e_#g_#m_#e_#n_#t of zero or more information  bytes.

The  allowed data segment sizes range between 32 and 4096 as

determined by the formula 32(2#8k#9) where k is a 3-bit  number.

The  packet  sequence numbers are likewise constrained to 3-

bits; i.e. counting proceeds modulo-8.

The control byte is partitioned into  three  fields  as

depicted below.

_________________________

|#- UNIX is a trademark of Bell Laboratories.

November 23, 1989

- 2 -

bit     7       6       5       4       3       2       1       0

t       t       x       x       x       y       y       y

The  _#t  bits  indicate  a  packet  type  and  determine  the

interpretation  to be placed on the _#x_#x_#x and _#y_#y_#y fields.  The

various interpretations are as follows:

_#t_#t      _#i_#n_#t_#e_#r_#p_#r_#e_#t_#a_#t_#i_#o_#n

00      control packet

10      data packet

11      `short' data packet

01      alternate channel

A data segment accompanies all  non-control  packets.   Each

transmitter  is constrained to observe the maximum data seg-

ment size established during initial synchronization by  the

receiver  that  it  sends  to.  Type 10 packets have maximal

size data segments.  Type 11, or `short', packets have  zero

or more data bytes but less than the maximum.  The first one

or two bytes of the data  segment  of  a  short  packet  are

`count'  bytes that indicate the difference between the max-

imum size and the number of bytes in the short segment.   If

the difference is less than 127, one count byte is used.  If

the difference exceeds 127, then the low-order seven bits of

the  difference are put in the first data byte and the high-

order bit is set as an indicator that the remaining bits  of

the  difference are in the second byte.  Type 01 packets are

never used by UUCP and need not be discussed in detail here.

The sequence number of a non-control packet is given by

the  _#x_#x_#x  field.   Control  packets  are not sequenced.  The

newest sequence number, excluding  duplicate  transmissions,

accepted  by  a  receiver is placed in the _#y_#y_#y field of non-

control packets sent to the `other' receiver.

There are no  data  bytes  associated  with  a  control

packet,  the  _#x_#x_#x field is interpreted as a control message,

and the _#y_#y_#y field is a value accompanying the  control  mes-

sage.   The  control messages are listed below in decreasing

priority.  That is, if several control messages  are  to  be

sent, the lower-numbered ones are sent first.

_#x_#x_#x     _#n_#a_#m_#e            _#y_#y_#y

1       CLOSE   n/a

2       RJ              last correctly received sequence number

3       SRJ             sequence number to retransmit

4       RR              last correctly received sequence number

5       INITC   window size

6       INITB   data segment size

7       INITA   window size

November 23, 1989

- 3 -

The CLOSE message  indicates  that  the  communications

channel  is  to  be  shut  down.  The RJ, or _#r_#e_#j_#e_#c_#t, message

indicates that the receiver has detected an  error  and  the

sender should retransmit after using the _#y_#y_#y field to update

the window.  This mode of retransmission is usually referred

to  as  a  `go-back-N'  procedure.   The  SRJ,  or _#s_#e_#l_#e_#c_#t_#i_#v_#e

_#r_#e_#j_#e_#c_#t, message carries with it the  sequence  number  of  a

particular  packet to be retransmitted.  The RR, or _#r_#e_#c_#e_#i_#v_#e_#r

_#r_#e_#a_#d_#y, message indicates that the receiver has  detected  no

errors;  the  _#y_#y_#y  field  updates  the sender's window.  The

INITA/B/C messages are used to set window and  data  segment

sizes.  Segment sizes are calculated by the formula 32(2#8yyy#9)

as mentioned above, and window sizes may range between 1 and

7.

Measurements of the protocol running  on  communication

links  at rates up to 9600 baud showed that a window size of

2 is optimal given a packet  size  greater  than  32  bytes.

This  means that the link bandwidth can be fully utilized by

the software.  For this reason the SRJ  message  is  not  as

important  as  it  might  otherwise  be.  Therefore the UNIX

implementations no longer generate or respond  to  SRJ  mes-

sages.   It  is mentioned here for historical accuracy only,

and one may assume that SRJ is no longer part of the  proto-

col.

_#M_#e_#s_#s_#a_#g_#e _#E_#x_#c_#h_#a_#n_#g_#e_#s

_#I_#n_#i_#t_#i_#a_#l_#i_#z_#a_#t_#i_#o_#n

Messages are exchanged between four  cooperating  enti-

ties:  two  senders  and two receivers.  This means that the

communication channel  is  thought  of  as  two  independent

half-duplex  data paths.  For example the window and segment

sizes need not be the same in each direction.

Initial synchronization is accomplished with two  3-way

handshakes:  two  each  of  INITA/INITB/INITC.   Each sender

transmits INITA messages repeatedly.  When an INITA  message

is received, INITB is sent in return.  When an INITB message

is received _#a_#n_#d an INITB message has  been  sent,  an  INITC

message  is  sent.   The INITA and INITB messages carry with

them the packet and window size that each receiver wants  to

use,  and  the  senders  are  supposed  to  comply.   When a

receiver has seen all three INIT messages,  the  channel  is

considered to be open.

It is possible to design  a  protocol  that  starts  up

using   fewer   messages  than  the  interlocked  handshakes

described above.  The  advantage  of  the  more  complicated

design  lies  in  its use as a research vehicle: the initial

handshake sequence is completely symmetric, a handshake  can

be initiated by one side of the link while the connection is

in use, and the software to do this can  utilize  code  that

November 23, 1989

- 4 -

would ordinarily be used only once at connection setup time.

These properties were used in experiments  with  dynamically

adjusted  parameters.   That  is attempts were made to adapt

the window and segment sizes to changes observed in  traffic

while a link was in use.  Other experiments used the initial

handshake  in a different way for  restarting  the  protocol

without  data loss after machine crashes.  These experiments

never worked well in the packet driver  and  basically  pro-

vided the impetus for other protocol designs.  The result as

far as UUCP is concerned  is  that  initial  synchronization

uses  the  two  3-way  handshakes, and the INIT messages are

ignored elsewhere.

_#D_#a_#t_#a _#T_#r_#a_#n_#s_#p_#o_#r_#t

After initial  synchronization  each  receiver  sets  a

modulo-8  incrementing  counter  R  to 0; each sender sets a

similar counter S to 1.  The value of R is always the number

of  the most recent correctly received packet.  The value of

S is always the first sequence number in the output  window.

Let  W  denote window size.  Note that the value of W may be

different for each sender.

A sender may transmit packets with sequence numbers  in

the  range  S  to  (S+W-1) mod-8.   At any particular time a

receiver expects arriving packets to  have  numbers  in  the

range  (R+1) mod-8  to  (R+W) mod-8.  Packets must arrive in

sequence number order are are only  acknowledged  in  order.

That  is, the `next' packet a receiver will acknowledge must

have sequence number (R+1) mod-8.

A receiver acknowledges  receipt  of  data  packets  by

arranging  for  the value of its R counter to be sent across

the channel where it will be used to update  an  S  counter.

This is done in two ways.  If data is flowing in both direc-

tions across a channel then each receiver's current R  value

is  carried in the _#y_#y_#y field of non-control packets.  Other-

wise  when  there  is  no  bidirectional  data  flow,   each

receiver's R value is transmitted across the link as the _#y_#y_#y

field of an RR control packet.

Error handling is up to the discretion of the receiver.

It  can ignore all errors in which case transmitter timeouts

must provide for retransmission.  The receiver may also gen-

erate  RJ error control packets.  The _#y_#y_#y field of an incom-

ing RJ message replaces the S value of the local sender  and

constitutes  a  request  for retransmission to start at that

sequence number.  The _#y_#y_#y field of an incoming  SRJ  message

selects a particular packet for retransmission.

The resemblance between the flow control  procedure  in

the  packet driver and that defined for X.25 is no accident.

The packet driver protocol  began  life  as  an  attempt  at

cleaning  up  X.25.   That  is  why,  for  example,  control

November 23, 1989

- 5 -

information is uniform in length (one byte), there is no RNR

message  (not  needed), and there is but one timeout defined

in the sender.

_#T_#e_#r_#m_#i_#n_#a_#t_#i_#o_#n

The CLOSE message is used to terminate  communications.

Software on either or both ends of the communication channel

may initiate termination.  In any case when one end wants to

terminate it sends CLOSE messages until one is received from

the other end or until a programmable limit on the number of

CLOSE  messages  is  reached.   Receipt  of  a CLOSE message

causes a CLOSE message to be sent.  In the UNIX  environment

it  also  causes  the  SIGPIPE or `broken pipe' signal to be

sent to the local process using the communication channel.

_#F_#r_#a_#m_#i_#n_#g

The term _#f_#r_#a_#m_#i_#n_#g is used to  denote  the  technique  by

which  the  beginning  and end of a message is detected in a

byte stream; _#e_#r_#r_#o_#r  _#c_#o_#n_#t_#r_#o_#l  denotes  the  method  by  which

transmission  errors  are  detected.  Strategies for framing

and error control depend upon additional  information  being

transmitted  along  with  the control byte and data segment,

and the choice of a particular strategy usually  depends  on

characteristics  of  input/output  devices  and transmission

media.

Several framing techniques are in used in support of PK

protocol  implementations, not all of which can be described

in detail here.  The technique used on  asynchronous  serial

lines will be described.

A six byte framing _#e_#n_#v_#e_#l_#o_#p_#e is  constructed  using  the

control  byte C of a packet and five other bytes as depicted

below.

<DLE><k><c0><c1><C><x>

The <DLE> symbol denotes the ASCII ctrl/P character.  If the

envelope  is  to  be followed by a data segment, <k> has the

value log#92#8(size)-4; i.e. 1 <#_ k <#_ 8.  If k  is  9,  then  the

envelope  represents  a  control  packet.  The <c0> and <c1>

bytes are the low-order and high-order bytes respectively of

0xAAAA  minus  a 16-bit checksum.  For control packets, this

16-bit checksum is the same as the control byte C.  For data

packets,  the  checksum  is calculated by the program below.

The <x> byte is the exclusive-or of  <k><c0><c1><C>.   Error

control  is  accomplished  by  checking  a  received framing

envelope for compliance with the definition, and comparing a

checksum function of the data segment with <c0><c1>.

This particular framing strategy assumes data  segments

are constant-sized: the `unused' bytes in a short packet are

actually transmitted.  This  creates  a  certain  amount  of

overhead  which  can  be  eliminated  by  a more complicated

November 23, 1989

- 6 -

framing technique.  The advantage of this strategy  is  that

i/o  devices  can  be  programmed  to  take advantage of the

constant-sized framing envelopes and data segments.

November 23, 1989

- 7 -

The checksum calculation is  displayed  below  as  a  C

function.   Note that the code is not truly portable because

the definitions of _#s_#h_#o_#r_#t and _#c_#h_#a_#r are not  necessarily  uni-

form  across  all machines that might support this language.

This code assumes that _#s_#h_#o_#r_#t and  _#c_#h_#a_#r  are  16  and  8-bits

respectively.

/* [Original document's version corrected to actual version] */

chksum(s,n)

register char *s;

register n;

{

register short sum;

register unsigned short t;

register short x;

sum = -1;

x = 0;

do {

if (sum<0) {

sum <<= 1;

sum++;

} else

sum <<= 1;

t = sum;

sum += (unsigned)*s++ & 0377;

x += sum^n;

if ((unsigned short)sum <= t) {

sum ^= x;

}

} while (--n > 0);

return(sum);

}

November 23, 1989